Test Information on Test Vehicle_v2.260831

- Required Document for the Test and Assessment

- Adult Occupant Protection

1.1 Knee mapping test data, where applicable The manufacturer shall submit the knee mapping data before the frontal offset deformable impact test, or within 6 weeks after the test at the latest. If it fails to do so, the penalty will be applied according to the protocol.

Description:

1.2 Front passenger data of the frontal full width impact test The manufacturer shall submit the front passenger data at least 1 week before the frontal full width impact test. If it is not willing or able to provide this data, TNCAP may perform the frontal full width test with an additional dummy.

Description:

1.3 Details about the front seats available for that model of vehicle Before the front seat whiplash test, the manufacturer shall submit the details about the front seats available for that model of vehicle and of any differences between the driver and front passenger’s seats in the basic level vehicle that might influence whiplash protection. If the manufacturer provides an unrealistically high quote or has excessively long lead times, the test will be scored zero points in accordance with TNCAP Section 1.4.4.3.2.

Description:

1.4 Evidence of the equivalence of the worst-case geometry assessment of the front seats Before the front seat whiplash test, the manufacturer shall provide evidence that the front seats of test variant are equivalent in terms of the worst-case geometry assessment; otherwise, the manufacturer will be asked to provide additional seat for assessment.

Description:

- Adult Occupant Protection

1.5 R point position and torso angle of the rear outboard seating positions of the test variant Before the rear seat whiplash test, the manufacturer shall provide the theoretical design data for R point position and torso angle of the two outboard seating positions of the test variant to prove that the outboard seating positions are symmetrical. If all the values are the same to within the tolerances, only one position needs to be measured. Otherwise, the two outboard seating positions will separately assessed.

Description:

1.6 Evidence of the vehicle with limited rear space, where necessary For vehicles with limited rear space, the manufacturer, before the assessment, shall provide evidence showing that the Child Restraint System (CRS) and/or child dummy cannot be installed without interference from the vehicle.

Description:

- Rescue Sheet The Rescue Sheet(s) (the model variant rated by TNCAP as well as other variants covered by the rating) shall be submitted before the assessment with additional information for database (i.e. links to OEM website, photo, etc.). If it is not submitted, the TNCAP website will disclose that the Rescue Sheet is not available for the model.

Description:

- Child Occupant Protection

2.1 Evidence of low risk deployment frontal airbag, where necessary Where the low risk deployment frontal airbag is fitted, the manufacturer shall provide convincing data before the assessment, which indicate that the frontal airbag can indeed be considered as low risk, and there must be information in the handbook indicating that this airbag can remain active when installing the rearward facing Child restraint system (CRS).

Description:

2.2 For vehicles with limited rear space, the manufacturer shall submit dynamic test data The manufacturer shall provide the dynamic test data with modified front seating positions before the assessment; otherwise, zero point will be awarded for the dynamic assessment.

Description:

| - Pedestrian Protection |
| --- |
| 3.1 Information about detection of pedestrian and deployable system Where an active pedestrian protection system is fitted, the manufacturer shall provide all necessary information related to the detection of pedestrians and deployment of the system (such as the specific details of the sensing, trigger and development systems, etc.) before the pedestrian test, including but not limited to: - General system description shall be provided which details the type of sensor system (pressure/acceleration etc.) and give details of any airbags, actuators and hinge mechanisms. - Active operating range for the system (km/h). - Human body model simulations for head impact time (HIT) of relevant pedestrian statures (6YO, 5th female, 50th male, 95th male). - System total response time (TRT) compared to HIT. - Confirmation of hardest to detect impactor (HTD). - Data from all necessary physical tests. - HIC data for an undeployed system at the lower operating limit. - Bonnet deflection analysis. |
| Description: |
| 3.2 Coordinates of all grid points derived from either CAE data or physical mark out, and HIC15 or colour data detailing the protection offered by the test variant at all headform grid locations. For headform impact test, the manufacturer shall provide the following data before any vehicle marking or testing begins: - Manufacturer grid marking coordinates relative to an identifiable location on the vehicle. - Predicted colour or HIC data clearly identifying defaulted points and blue points. - Justification for all blue points. - In case there are two blue zones next to each other, details of which point(s) form a blue zone. |
| Description: |
| 3.3 Evidence for the location and structure of symmetrical grid points in front of the vehicle For the legform impact test, if the structure of symmetrical grid points on both sides of the vehicle are different, the manufacturer should provide proof before the start location is selected and any nominations are made. |
| Description: |
| 3.4 List of parts required for the pedestrian protection test The manufacturer shall provide the list of parts required for the test before the pedestrian protection test. However, if the parts are quoted at unrealistically high prices or with excessively long lead times, the test will be scored zero points in accordance with TNCAP Section 1.4.4.3.2. |
| Description: |
| (IV) Safety Assist |  |
| 4.1 For the evaluation of Driver State Monitoring, the manufacturer must provide a dossier containing a detailed technical assessment. The dossier should contain: - Technical detail about the system, to fully understand its functionality, relevant components, and intended availability. - Test procedures, criteria and limits by which the performance of the system was verified. - If available, the dossier should summarize the findings from real-world or simulated real-world evaluations. |  |
| Description: |  |

- Specification Information Table of the Test Vehicle

| - Basic Information of the Vehicle |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vehicle Make |  |  |  | Model Name |  |  |  | Test Model |  |  |  |
| Vehicle Length | mm |  |  | Vehicle Width |  | mm |  | Number of Doors |  |  |  |
| Unladen Kerb Mass | kg (full fuel tank) |  |  | Front Axle Weight |  | kg |  | Maximum Permitted Laden Mass |  | kg (full fuel tank) |  |
|  |  |  |  | Rear Axle Weight |  | kg |  |  |  |  |  |
| Tire Specification (Enter based on actual vehicle) | Brand |  | Model |  | Dimension |  | Speed Code |  | Load Capability Index |  | Cold Tire Pressure (unit) |
| Front Axle |  |  |  |  |  |  |  |  |  |  |  |
| Rear Axle |  |  |  |  |  |  |  |  |  |  |  |
| Geometric Inspection (Unit: degree & arc minute) | Left Toe-in (with tolerances) |  | Right Toe-in (with tolerances) |  | Left Camber (with tolerances) |  | Right Camber (with tolerances) |  | Left Caster (with tolerances) |  | Right Caster (with tolerances) |
| Front Axle |  |  |  |  |  |  |  |  |  |  |  |
| Rear Axle |  |  |  |  |  |  |  |  |  |  |  |
| Recommended value of air pressure for tire in proper load state | ☐ Recommended Value Description: |  |  |  |  |  | ☐ Figure: |  |  |  |  |
| Fender Height with no Load | The height of left front fender from ground is mm, The height of left rear fender from ground is mm. The height of right front fender from ground is mm, The height of right rear fender from ground is mm. |  |  |  |  |  |  |  |  |  |  |
| Engine Type | ☐ ICE | ☐ 92 Unleaded Gasoline ☐ 95 Unleaded Gasoline ☐ Diesel Fuel tank capacity： Liter |  |  |  |  |  |  |  |  |  |
|  | ☐ Electric | ☐ Pure Electric ☐ Hybrid (Please declare the fuel type above) ☐ Fuel cell: Fuel capacity: liter, Fuel type: . (Please attach the electric system specifications and charging operation documents.) |  |  |  |  |  |  |  |  |  |
|  | ☐ Other: |  |  |  |  |  |  |  |  |  |  |
| Spare Tire | ☐ Available ☐ Not available |  |  |  |  |  |  |  |  |  |  |
| Accessory Tools | ☐ Available ☐ Not available |  |  |  |  |  |  |  |  |  |  |

| - Vehicle Body Coordinate Structure and Design Value |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference Point | Position Description |  | Coordinate Value (X, Y, Z) |  | Diagram Description |  |  | Diagram of Coordinate Axis Direction |  |  |  |
| NO.01 | Fixing point above the striker of the left front door |  | NO.01 X: Y: Z: |  |  |  |  |  |  |  |  |
| NO.02 | Fixing point of the hinge rod of the left front door |  | X: Y: Z: |  | NO.02 |  |  |  |  |  |  |
| NO.03 | Fixing point of the front left seat leg of driver seat |  | X: Y: Z: |  | NO.04 NO.03 |  |  |  |  |  |  |
| NO.04 | Fixing point of the front right seat leg of driver seat |  | X: Y: Z: |  |  |  |  |  |  |  |  |
| NO.05 | Fixing point of the rear left seat leg of driver seat |  | X: Y: Z: |  |  |  |  |  |  |  |  |
| NO.06 | Fixing point of the rear right seat leg of driver seat |  | X: Y: Z: |  |  |  |  |  |  |  |  |
| NO.07 | Fixing point of the front left seat leg of the front passenger seat |  | X: Y: Z: |  | NO.05 NO.06 NO.08 NO.07 |  |  |  |  |  |  |
| NO.08 | Fixing point of the front right seat leg of the front passenger seat |  | X: Y: Z: |  |  |  |  |  |  |  |  |
| NO.09 | Fixing point of the rear left seat leg of the front passenger seat |  | X: Y: Z: |  | NO.09 NO.10 |  |  |  |  |  |  |
| NO.10 | Fixing point of the rear right seat leg of the front passenger seat |  | X: Y: Z: |  |  |  |  |  |  |  |  |
| Please provide the coordinate reference point on the plane of the vehicle body. If there are other reference points other than the above, please also provide relevant diagram description. |  |  |  |  |  |  |  |  |  |  |  |
| R point coordinate of driver seat |  |  | X: |  |  |  | Y: |  |  | Z: |  |
| R point coordinate of front passenger seat |  |  | X: |  |  |  | Y: |  |  | Z: |  |
| The R point coordinate at the 50th percentile at the outside of the 2nd row |  | Left | X: |  |  |  | Y: |  |  | Z: |  |
|  |  | Right | X: |  |  |  | Y: |  |  | Z: |  |
| The R point coordinate at the 50th percentile at the outside of the 3rd row |  | Left | X: |  |  |  | Y: |  |  | Z: |  |
|  |  | Right | X: |  |  |  | Y: |  |  | Z: |  |
| - Function Information of Adjustments |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Driver Seat |  | Front Passenger Seat |  |  | The Outside Of The 2nd Row Seat |  | The Outside Of The 3rd Row Seat |
| Seat Fore/Aft Adjustment * |  |  |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |
| Seat Base Tilt Adjustment |  |  |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |
| Seat Height Adjustment * |  |  |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |
| Seat Lumbar Support * |  |  |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |
| Seat Back Angle Adjustment * |  |  |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |
| Upper Seat Back Adjustment |  |  |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |
| Side Bolsters |  |  |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |
| Seat Cushion Tilt Adjustment* |  |  |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |
| Seat Cushion Height Adjustment |  |  |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |
| Cushion Extension |  |  |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |
| Leg Support System |  |  |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |
| Movable Arm-Rests* |  |  |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |
| Seat Lateral Adjustment |  |  |  | － |  | ☐ |  |  | ☐ |  | ☐ |
| Seat Facing Adjustment |  |  |  | － |  | ☐ |  |  | ☐ |  | ☐ |
| Head Restraint Height Adjustment * |  |  |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |
| Head Restraint Tilt Adjustment * |  |  |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |
| Seat Belt Anchorage Adjustment * |  |  |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |
| Steering Wheel - Horizontal * |  |  |  | ☐ |  | Steering Wheel – Vertical |  |  |  |  | ☐ |
| Accel Pedal Adjustment * |  |  |  | ☐ |  | Brake Pedal Adjustment |  |  |  |  | ☐ |
| Clutch Pedal Adjustment |  |  |  | ☐ |  | Parking Brake Pedal Adjustment |  |  |  |  | ☐ |
| Others Adjustments |  |  |  |  |  |  |  |  |  |  |  |

* Primarily Adjustment Device

| - Manufacturer's Design Position For Crash And Whiplash Testing (Null If Not Available) |  |  |  |
| --- | --- | --- | --- |
| Offset Deformable Barrier Frontal Impact | Driver Seat (H-III 50M) | Front Passenger Seat (H-III 50M) | Q6/Q10 |
| Seat Fore/Aft Adjustment | Mid position. Position for the 95th percentile male: notch/mm rearward form foremost position. | Mid position. Position for the 95th percentile male: notch/mm rearward form foremost position. | Position for Q6: notch/mm rearward form foremost position. Position for Q10: notch/mm rearward form foremost position. |
| Seat Cushion Tilt Adjustment | ☐ notch/degree upper from flattest position. ☐Others: . | ☐ notch/degree upper from flattest position. ☐Others: . | － |
| Seat Back Angle Adjustment (Declare one) | ☐Torso Angle ∘ ☐ notch/degree rearward from foremost position. ☐Others: . | ☐Torso Angle ∘ ☐ notch/degree rearward from foremost position. ☐Others: . | ☐Torso Angle ∘ ☐ notch/degree rearward from foremost position. ☐Others: . |
| Movable Arm-Rests | notch/mm upper form lowest position. | notch/mm upper form lowest position. | Stowed position. |
| Seat Lumbar Support | ☐ notch/mm forward from rearmost position. ☐Others: . | ☐ notch/mm forward from rearmost position. ☐Others: . | ☐ notch/mm forward from rearmost position. ☐Others: . |
| Seat Belt Anchorage | notch/mm lower form highest position. | notch/mm lower form highest position. | As recommended in vehicle handbook for CRS installation |
| Accel Pedal Adjustment | ☐ notch/mm upper form lowest position. ☐ Others: . | － | － |
| Brake pedal retraction mechanism | ☐Fitted (☐Engine should be run for seconds ☐ Engine does not need to be ran) ☐Not fitted |  |  |
| Others (Please elaborate) |  |  |  |

| Full Width Frontal Impact | Driver Seat (H-III 5F) | Front Passenger Seat (H-III 5F) | The Outside of 2nd Row (H-III 5F) |
| --- | --- | --- | --- |
| Seat Fore/Aft Adjustment | notch/mm rearward form foremost position. | notch/mm rearward form foremost position. | notch/mm rearward form foremost position. |
| Seat Cushion Tilt Adjustment | ☐ notch/degree upper from flattest position. ☐Others: . | ☐ notch/degree upper from flattest position. ☐Others: . | ☐ notch/degree upper from flattest position. ☐Others: . |
| Seat Height Adjustment | notch/mm upper form lowest position. | notch/mm upper form lowest position. | notch/mm upper form lowest position. |
| Seat Back Angle Adjustment (Declare one) | ☐Torso Angle ∘ ☐ notch/degree rearward from foremost position. ☐Others: . | ☐Torso Angle ∘ ☐ notch/degree rearward from foremost position. ☐Others: . | ☐Torso Angle ∘ ☐ notch/degree rearward from foremost position. ☐Others: . |
| Movable Arm-Rests | notch/mm upper form lowest position. | notch/mm upper form lowest position. | Stowed position. |
| Head Restraint Tilt Adjustment | ☐Non-locking, degree rearward form foremost position. ☐Locking, degree rearward form foremost position. ☐Automatically Adjusting | ☐Non-locking, degree rearward form foremost position. ☐Locking, degree rearward form foremost position. ☐Automatically Adjusting | ☐Non-locking, degree rearward form foremost position. ☐Locking, degree rearward form foremost position. ☐Automatically Adjusting |
| Seat Lumbar Support | ☐ notch/mm forward from rearmost position. ☐Others: . | ☐ notch/mm forward from rearmost position. ☐Others: . | According to protocol. |
| Seat Belt Anchorage | notch/mm lower form highest position. | notch/mm lower form highest position. | notch/mm lower form highest position. |
| Steering Wheel - Horizontal | ☐ notch/mm rearward form foremost position. ☐Others: . | － | － |
| Accel Pedal Adjustment | ☐ notch/mm upper form lowest position. ☐ Others: . | － | － |
| Brake pedal retraction mechanism | ☐Fitted (☐Engine should be ran for seconds ☐ Engine does not need to be ran) ☐Not fitted |  |  |
| Others (Please elaborate) |  |  |  |

| Side Impact Mobile Deformable Barrier Oblique Pole Side Impact | Driver Seat (WorldSID 50M) | Q6/Q10 |
| --- | --- | --- |
| Seat Fore/Aft Adjustment | According to protocol. | notch/mm rearward form foremost position. |
| Seat Back Angle Adjustment (Declare one) | ☐Torso Angle ∘ ☐ notch/degree rearward from foremost position. ☐Others: . | ☐Torso Angle ∘ ☐ notch/degree rearward from foremost position. ☐Others: . |
| Movable Arm-Rests | notch/mm upper form lowest position. | Stowed position. |
| Seat Belt Anchorage | notch/mm lower form highest position. | As recommended in vehicle handbook for CRS installation |
| Others (Please elaborate) |  |  |
| WorldSID Rib Angle | degree(positive upwards) |  |
| Reference for protocols on WorldSID rib angle. 3.3.5.2.5.1/3.4.5.3.4.1 Adjust the dummy until the thorax tilt sensor coincides with the angle specified by the manufacturer. 3.3.5.2.5.2/3.4.5.3.4.2 If the rib angle is not specified by the manufacturer and the seat back is 23° ± 1°, adjust the dummy until the thorax tilt sensor reads −2° (spine flexion) ± 1° 3.3.5.2.5.3/3.4.5.3.4.3 If no rib angle is specified and the seat back angle is not 23° ± 1°, no further adjustment Version 8.3 5th December 2023 17 of rib angle is required. |  |  |
| Head Protection Device, HPD |  |  |
| The Pressure for Side Airbags | Front Seat: psi, ☐ Curtain airbags ☐ Seat mounted head protection devices ☐ Not fitted Rear Seat: psi, ☐ Curtain airbags ☐ Seat mounted head protection devices ☐ Not fitted |  |

| Front Seat Whiplash |  |  |  |
| --- | --- | --- | --- |
| Seat Belt | ☐Use generic seat belts anchorage mounting. ☐Use vehicle specific belts and geometry (Please provide the installation mounting and the coordinate information below). |  |  |
| Seat Belt Upper Fixed Point | X: | Y: | Z: |
| Seat Belt Lower Fixed Point | X: | Y: | Z: |
| Seat Belt Buckle Fixed Point | X: | Y: | Z: |
| Seat Belt Pretensioner | ☐Not fitted ☐Fitted, do not need to trigger in the test. ☐Fitted, need to trigger in the test, time to fire: ms |  |  |
| Active Head Restraint | ☐Not fitted ☐Re-active head restraint ☐Pro-active head restraint, time to fire: ms |  |  |
| Z Axis Coordinate of Heel Plane Height |  |  |  |
| Seat Rail Angle |  |  |  |

| Rear Seat Whiplash | The Outside of 2nd Row Seat | The Outside of 3rd Row Seat |
| --- | --- | --- |
| Seat Fore/Aft Adjustment | notch/mm rearward form foremost position. | notch/mm rearward form foremost position. |
| Seat Base Tilt Adjustment | ☐ notch/degree upper from flattest position. ☐Others: . | ☐ notch/degree upper from flattest position. ☐Others: . |
| Seat Height Adjustment | notch/mm upper form lowest position. | notch/mm upper form lowest position. |
| Seat Lumbar Support | ☐ notch/mm forward from rearmost position. ☐Others: . | ☐ notch/mm forward from rearmost position. ☐Others: . |
| Seat Back Angle Adjustment | ☐Torso Angle ∘ ☐ notch/degree rearward from foremost position. ☐Others: . | ☐Torso Angle ∘ ☐ notch/degree rearward from foremost position. ☐Others: . |
| Head Restraint Height Adjustment | ☐Non-locking, mm upper form lowest position. ☐Locking, notch upper form lowest position. ☐Automatically Adjusting | ☐Non-locking, mm upper form lowest position. ☐Locking, notch upper form lowest position. ☐Automatically Adjusting |
| Head Restraint Tilt Adjustment | ☐Non-locking, degree rearward form foremost position. ☐Locking, degree rearward form foremost position. ☐Automatically Adjusting | ☐Non-locking, degree rearward form foremost position. ☐Locking, degree rearward form foremost position. ☐Automatically Adjusting |
| Torso Angle of the 50th Percentile Male | Left: . Right: . | Left: . Right: . |
| Centreline of a seating position (C/LO) | ☐Marking according to protocol. ☐Marking according to coordinate: Left X: , Right X: . ☐Others (Please elaborate) | ☐Marking according to protocol. ☐Marking according to coordinate: Left X: , Right X: . ☐Others (Please elaborate) |

* Only the main adjustment devices are listed. If other adjustment devices need to be declared, please submit separate documents to explain. If not declared, they will be installed in accordance with protocol.

| - Setting Parameters of Pedestrian Protection Test |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. The Normal Ride Attitude with the reference of the vertical (Z) position of any marks, holes, surfaces and identification signs on the vehicle body, above the ground. |  |  |  |  |  |  |  |  |
| Fender Height at the Normal Ride Attitude | The height of left front fender from ground is mm, The height of left rear fender from ground is mm. The height of right front fender from ground is mm, The height of right rear fender from ground is mm. |  |  |  |  |  |  |  |
| Fender Height at 40 km/h | ☐ With adjustable suspension, please provide information about the height from the ground. ☐ Without adjustable suspension, there is no need to provide information in this field. The height of left front fender from ground is mm, The height of left rear fender from ground is mm. The height of right front fender from ground is mm, The height of right rear fender from ground is mm. |  |  |  |  |  |  |  |
| Other |  |  |  |  |  |  |  |  |
| 2. Vehicle Body Coordinate Structure and Design Value |  |  |  |  |  |  |  |  |
| Reference Point | Coordinate Value (X, Y, Z) |  |  |  |  | Diagram of Coordinate Axis Direction |  |  |
| NO.01 | X: | Y: |  |  | Z: |  |  |  |
| NO.02 | X: | Y: |  |  | Z: |  |  |  |
| NO.03 | X: | Y: |  |  | Z: |  |  |  |
| NO.04 | X: | Y: |  |  | Z: |  |  |  |
| NO.05 | X: | Y: |  |  | Z: |  |  |  |
| Please provide the coordinate reference point on the front 1/3 area of the vehicle (hood, engine bay, etc.) If there are other reference points other than the above, please also provide relevant diagram description. |  |  |  |  |  |  |  |  |
| 3. Vehicle Mark |  |  |  |  |  |  |  |  |
| Grid marking coordinates | ☐ Provide CAE data. (Please refer to “Other Remarks” below) ☐ Provide physical mark out. |  |  |  |  |  |  |  |
| Grid points for comparison | The grid origin C0,0 |  |  | X: |  |  | Y: | Z: |
|  | 2 points on WAD2100 |  | A11, | X: |  |  | Y: | Z: |
|  |  |  | A11, | X: |  |  | Y: | Z: |
| Legform Impact Test | ☐ The height of the lower bumper reference line is less than 425 mm, and the legform impactor is adopted. ☐ The height of the lower bumper reference line is more than 500 mm, and the upper legform impactor is adopted. ☐ The height of the lower bumper reference lines is between 425~500 mm, adopt ☐ legform impactor ☐ upper legform impactor |  |  |  |  |  |  |  |
| Deployable systems | ☐ Deployable systems ☐ Other: ☐ Not available |  |  |  |  |  |  |  |
| Other Remarks | All the grid marking coordinates of the CAE data or the physical mark out: - Two identifiable points on the vehicle centreline (Y=0). (For example, the emblem in front, the camera on the windshield, etc.) - The coordinates of the corner reference points, the corner of bumper, and the outermost ends of the bumper beam/lower rails/cross beam structures relative to the identifiable positions on the vehicle. - The coordinates of the headform grid points and the upper legform WAD775mm grid points relative to the identifiable positions on the vehicle. |  |  |  |  |  |  |  |

| - Active Safety System Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Autonomous Emergency Braking System ☐Not equipped | 1. System Specification and Execution Requirement |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ☐City | System Name |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Autonomous Emergency Braking(AEB) | Speed Range |  |  | Max Value |  | km/h |  |  |  | Min. Value | km/h |  |
|  | ☐ Inter-Urban | System Name |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ☐ Autonomous Emergency Braking(AEB) | Speed Range |  |  |  | Max Value |  | km/h |  |  | Min. Value |  | km/h |
|  |  | ☐ Forward Collision Warning(FCW) | Speed Range |  |  |  | Max Value |  | km/h |  |  | Min. Value |  | km/h |
|  | ☐Vulnerable Road User | System Name |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ☐ Autonomous Emergency Braking(AEB) | Speed Range |  |  |  | Max Value |  | km/h |  |  | Min. Value |  | km/h |
|  |  | ☐ Autonomous Emergency Braking and Forward Collision Warning (AEB & FCW) |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Test scenarios where the function is enabled/active: ☐Car-to-Pedestrian Farside Adult (CPFA) ☐Car-to-Pedestrian Nearside Adult(CPNA-25, CPNA-75);☐Day ☐Night ☐ Car-to-Pedestrian Nearside Child(CPNC) ☐ Car-to-Pedestrian Longitudinal Adult(CPLA-25, CPLA-50);☐Day ☐Night ☐ Car-to-Bicyclist Nearside Adult(CBNA-50) ☐ Car-to-Bicyclist Longitudinal Adult(CBLA-25, CBLA-50) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pedestrian/ Vulnerable Road User Protection System (i.e. a deployable pedestrian/ VRU protection system) | ☐ Equipped (Please describe disable method in the following blank space);☐Equipped and already disable;☐Not equipped |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Disable Method Description: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2.System Setting Method and Description |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Speed Control* | ☐Equipped with speed limit device |  | ☐Can be used during assessment execution;☐Not available |  |  |  |  |  |  |  |  |  |  |
|  |  | ☐Equipped with Cruise Control |  | ☐Can be used during assessment execution;☐Not available |  |  |  |  |  |  |  |  |  |  |
|  | *Only confirm whether it will affect the Autonomous Emergency Braking System of the vehicle to be tested, instead of specifying the assessment execution method. |  |  |  |  |  |  |  |  | ☐Yes; ☐No |  |  |  |  |
|  | Timing of FCW Collision Warning | ☐ Can be set (Please describe setting method as below); ☐Cannot be set |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Setting Method Description: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Timing of AEB Braking Application | ☐Can be set (Please describe setting method as below); ☐Cannot be set |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Setting Method Description: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | If requested by the vehicle manufacturer, drive a maximum of 100km on a mixture of urban and rural roads with other traffic and roadside furniture to ‘calibrate’ the sensor system. Avoid harsh acceleration and braking. |  |  |  |  |  |  |  |  | ☐Drive on test track to ‘calibrate’ the sensor system ☐Drive on a mixture of urban and rural roads to ‘calibrate’ the sensor system ☐No need to execute |  |  |  |  |
|  | If requested by the OEM an initialisation run may be included before every test run.** |  |  |  |  |  |  |  |  |  | ☐Yes ☐No |  |  |  |
|  | **Before every test run, drive the VUT around a circle of maximum diameter 30m at a speed less than 10km/h for one clockwise lap followed by one anticlockwise lap, and then manoeuvre the VUT into position on the test path. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Deactivate Function of AEB and/or FCW |  |  |  |  |  |  |  |  |  | ☐Cannot be deactivate ☐ Can be manually deactivate (Please describe the deactivate method in the following blank) |  |  |  |
|  | Deactivate Method Description: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Supplementary warning for the FCW system | In addition to the required audiovisual warning, a more sophisticated warning like head-up display, belt jerk, brake jerk or any other haptic feedback. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ☐ Equipped;☐Not equipped |  |  | Warning Method: |  |  |  |  |  |  |  |  |  |
|  | Reversible pre-tensioning of the belt in the pre-crash phase | ☐Equipped;☐Not equipped |  |  |  |  |  |  |  |  |  |  |  |  |
|  | AEB Non-Availability State | Is the AEB system subject to a limit on the number of activations, and upon reaching this limit, does the system provide a visual warning (e.g., indicator or symbol) to inform the driver that the system is currently unavailable or deactivated. ☐Equipped;☐Not equipped |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Restoring Method Description: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Attention: ☐Applicant understands that when the active safety test item is conducted by the laboratory according to the assessment standard, it will inevitably collide, so the body damage caused by it cannot be completely avoided. If it is possible to carry out the continuous assessment item after the damage, it will continue to carry out, and the tested vehicle will be returned to the applicant after all active safety items are completed. ☐In case that the tested vehicle is unable to continue the test rating due to physical damage during the active safety test, the implementing agency, the laboratory and the tested manufacturer will determine the responsibility through negotiation, and bear the relevant expenses according to the negotiation results. |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Lane Support Systems ☐Not equipped | 1. System Specification and Execution Requirement |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | System Name |  |  |  |  |  |  |  |  |  |  |
|  | Warning Signal |  | ☐Audible;☐Visual;☐Haptic;☐Other: |  |  |  |  |  |  |  |  |
|  | Equipped with ESC system** |  | ☐Equipped;☐Not equipped |  |  |  |  |  |  |  |  |
|  | ＊＊ The tested vehicle shall be equipped with the following requirements: - Electronic Stability Control according to UN R140 standard of the United Nations, or - Electronic Stability Control specified in Section 5.6 of Vehicle Safety Testing Directions 42-3 announced by the Ministry of Transportation and Communications, or - Electronic Stability Control specified in Vehicle Safety Testing Directions 85 announced by the Ministry of Transportation and Communications |  |  |  |  |  |  |  |  |  |  |
|  | ☐ Lane Departure Warning (LDW) |  |  |  | Speed Range | Max Value |  |  | km/h | Min. Value | km/h |
|  | ☐Lane Keeping Assist (LKA) |  |  |  | Speed Range | Max Value |  |  | km/h | Min. Value | km/h |
|  | ☐Emergency Lane Keeping (ELK) |  |  |  | Speed Range | Max Value |  |  | km/h | Min. Value | km/h |
|  | In case of Lane Keeping Assist (LKA), Emergency Lane Keeping (ELK) |  | ☐ The vehicle manufacturer shall provide information describing the location when the closed loop path and/or speed control shall be ended*** ☐Otherwise for each lateral velocity, two calibration runs shall be performed in order to determine when the system activates. Compare steering wheel torque, vehicle speed or yaw rate of both runs and determine where there is a notable difference that identifies the location of intervention |  |  |  |  |  |  |  |  |
|  | ***Document is provided describing the location of LKA and/or ELK intervene, so as not to interfere with the system intervention for each test. |  |  |  |  |  |  |  |  |  |  |
|  | 2.System Setting Method and Description |  |  |  |  |  |  |  |  |  |  |
|  | Lane Departure Warning (LDW) |  | ☐Can be set (Please describe setting method in the following blank);☐Cannot be set |  |  |  |  |  |  |  |  |
|  | Setting Method Description: |  |  |  |  |  |  |  |  |  |  |
|  | Lane Keeping Assist (LKA) |  | ☐Can be set (Please describe setting method in the following blank);☐Cannot be set |  |  |  |  |  |  |  |  |
|  | Setting Method Description: |  |  |  |  |  |  |  |  |  |  |
|  | If requested by the vehicle manufacturer, drive a maximum of 100km on a mixture of urban and rural roads with other traffic and roadside furniture to ‘calibrate’ the sensor system. Avoid harsh acceleration and braking. |  |  |  |  |  |  | ☐Drive on test track to ‘calibrate’ the sensor system ☐Drive on a mixture of urban and rural roads to ‘calibrate’ the sensor system ☐No need to execute |  |  |  |
|  | If requested by the OEM an initialisation run may be included before every test run.**** |  |  |  |  |  |  | ☐Yes ☐No |  |  |  |
|  | ****Before every test run, drive the VUT around a circle of maximum diameter 30m at a speed less than 10km/h for one clockwise lap followed by one anticlockwise lap, and then manoeuvre the VUT into position on the test path. |  |  |  |  |  |  |  |  |  |  |
| Speed Assistance Systems ☐Not equipped | System Specification and Execution Requirement |  |  |  |  |  |  |  |  |  |  |
|  | Speed Assistance Systems Information |  |  | System Name |  |  |  |  |  |  |  |
|  |  |  |  | Active braking |  |  | ☐Equipped;☐ Not equipped |  |  |  |  |
|  | Speed Assistance Systems Function |  |  | ☐ Speed Limit Information Function (SLIF)***** ☐ Speed Limitation Function (SLF)***** ☐ Intelligent Speed Assistance (ISA) ☐ Intelligent Adaptive Cruise Control (iACC) |  |  |  |  |  |  |  |
|  | ***** This function is available as an optional feature and can be selected in combination with other options. |  |  |  |  |  |  |  |  |  |  |
|  | Speed Limit Information Function(SLIF) | Display Method |  | ☐The speed limit information must be shown at any time ☐ The speed limit information must be accessible at any time with a simple operation (Please describe operation method in the following blank) |  |  |  |  |  |  |  |
|  |  |  |  | Operation Method: |  |  |  |  |  |  |  |
|  |  | Camera or map based or a combination of both |  | ☐Camera;☐ Map;☐Camera and Map |  |  |  |  |  |  |  |
|  |  |  |  | ☐Equipped;☐Not equipped with subsign recognition |  |  |  |  |  |  |  |
|  |  |  |  | ☐Updated;☐Not updated If map-based data must be updated frequently (at least quarterly) and automatically for the first six years, without user action. (ex. sending DVD/USB to customer allowed). |  |  |  |  |  |  |  |
|  |  | Supplementary Warning Function |  | ☐Audible;☐Haptic;☐Head-Up Display;☐Other (Please describe in the following blank) |  |  |  |  |  |  |  |
|  |  |  |  | Description of other supplementary warning functions: |  |  |  |  |  |  |  |
|  |  | Mobile devices connected to the vehicle network |  | ☐The speed limit information could be shown on mobile devices connected to the vehicle network. ☐The speed limit information map-based data could be provided by mobile devices connected to the vehicle network. |  |  |  |  |  |  |  |

| Occupant Status Monitoring ☐Not equipped | 1.System Specification and Execution Requirement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | System Name |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Warning Type | The 1st Row |  |  |  |  | Rear 1st Row |  |  | Rear 2nd Row |  |  |  | Rear 3rd Row |
|  | Visual Signal | ☐ |  |  |  |  | ☐ |  |  | ☐ |  |  |  | ☐ |
|  | Audible Signal | ☐ |  |  |  |  | ☐ |  |  | ☐ |  |  |  | ☐ |
|  | Seating Positions of SBR |  |  |  | ☐Seating Positions of Occupant Detection System |  |  |  |  |  | ☐Seating Positions of Removable Seats |  |  |  |
|  | The 1st Row |  |  |  | The 1st Row |  |  |  |  |  | The 1st Row |  |  |  |
|  | Rear 1st Row |  |  |  | Rear 1st Row |  |  |  |  |  | Rear 1st Row |  |  |  |
|  | Rear 2nd Row |  |  |  | Rear 2nd Row |  |  |  |  |  | Rear 2nd Row |  |  |  |
|  | Rear 3rd Row |  |  |  | Rear 3rd Row |  |  |  |  |  | Rear 3rd Row |  |  |  |
|  | Marking method :■For seating position with function;□For seating position without function;〼Not equipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Driver’s Seat |  | Front seat passengers |  | Rear 1st Row | Rear 2nd Row |  |  | Rear 3rd Row | Remark |  |
|  | Equipped with DSM |  |  | ☐ |  |  |  |  |  |  |  |  | The samples of triggering condition are as follows: - The car has reached a forward speed of ○○km/h - The engine has been running for ○○ seconds - Turn on ignition switch - “Forward Motion” for ○○ meters |  |
|  | Equipped with multiple signal |  |  | ☐ |  | ☐ |  | ☐ | ☐ |  |  | ☐ |  |  |
|  | Equipped with visual signal |  |  | ☐ |  | ☐ |  | ☐ | ☐ |  |  | ☐ |  |  |
|  | Triggering condition of visual signal |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Equipped with initial audible signal |  |  | ☐ |  | ☐ |  | ☐ | ☐ |  |  | ☐ |  |  |
|  | Triggering condition of initial audible signal |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Equipped with final audible signal |  |  | ☐ |  | ☐ |  | ☐ | ☐ |  |  | ☐ |  |  |
|  | Triggering condition of final audible signal |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2.System Setting Method and Description |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Seat Belt Reminder Device Other functions ☐Not equipped |  | ☐Manual switch off function at rear row position (Please describe switch off method in the following blank) |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Switch off operation description: |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | ☐Manual switch off air bag function at front seat passenger (Please describe switch off method in the following blank) |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Switch off operation description: |  |  |  |  |  |  |  |  |  |  |  |

| Blind Spot Assist Systems ☐Not equipped | System Specification and Execution Requirement |  |  |
| --- | --- | --- | --- |
|  | Blind Spot Assist Systems Information | System Name |  |
|  |  | Centre of the 95th percentile eyellipse | Distance from the front wheel center to the centre of the 95th percentile eyellipse: m |
|  | Blind Spot Assist Systems Function | Blind Spot Detection | ☐Equipped;☐Not equipped |
|  |  | Blind Spot Visualization | ☐Equipped;☐Not equipped |
|  | Warning Function | ☐Audible;☐Visual;☐Real-Time Camera View;☐ Other(Please describe in the following blank) |  |
|  |  | Description of other warning functions: |  |

- Attached File

- The power type of the test vehicle is electric/hybrid drive, and electrical system specifications and charging operation documents shall be provided. REESS Type; Working Voltage; V. REESS Capacity; kWh. Chemical Ingredients of Electrolyte capacity; Total Volume: L; Specific Gravity:; Color:. Manual Service Disconnect (MSD); ☐ Not available ☐ Available, position:. Automatic Disconnect; ☐ Not available ☐ Available, position:. Automatic Disconnect Operation Indicator; ☐ Not available ☐ Available, position:. Charging status of REESS normal operation situation; ☐ SOC: % ~ % or ☐ Voltage: V ~ V. Relevant power system components. Power system layout chart. Power system components*2; Traction motor. REESS*3. Electric energy conversion system. Electrical converter. Auxiliaries for running. Coupling system for charging system. Other. *1. For fuel cells, please provide other relevant information such as fuel type and storage capacity. *2. Indicate respective locations (e.g., inside/outside the cabin) and attach drawings and photographs. Additionally, specify where to check for protection from electric shock *3. Indicate where they are fixed and the method of fixation. - Requirements for protection from electric shock offered by the manufacturer: ☐ Measuring Isolation Resistance: Please provide the working voltage, current type and measurement location of each power system components, and assist in wiring. ☐ Measuring the Residual Voltage: Please provide the working voltage information and measurement location of high voltage components, and assist in wiring. ☐ Measuring the Residual Energy: Please define the capacitance information and measurement location of each capacitor and assist in wiring. - If there are any collision avoidance related systems, please provide the closing mode to avoid operation in collision test. - If the device has energy-saving features such as automatic power shut-off or a battery management system (BMS), please provide relevant information to prevent the automatic power shut-off or related systems from being used during crash tests. - Please provide the disassembly method of steering wheel and airbag. - If the seat is electric, please provide the wiring pin position definition diagram. - If the head restraint is pro-active, please provide control mode or trigger signal information. - If the design position setting of Q6/Q10 moving parts is different, please provide separate setting positions. - If there are other body coordinate reference points, please provide relevant diagram description. - During the pedestrian protection test, the front cover needs to be in a dynamic development state. Please provide the necessary information on the trigger system, specific details of the sensing, trigger and deployment systems. - Please provide the pedestrian protection test body marking grid.

- Whether the vehicle is fitted with Automatic Door Locking (ADL) function: ☐ Without ADL. ☐ ADL is fitted as standard and by default always ON. ☐ ADL is not fitted as standard, or not by default always ON, but fitted to the test variant. - According to the requirements of protocol 2.1.6.2.4, provide the score for all grid points per test speed for AEB City. AEB City : Predicted score for CCRs-AEB Test Speed; -50 %; -75 %; 100 %; +75 %; +50 %. 10 km/h. 15 km/h. 20 km/h. 25 km/h. 30 km/h. 35 km/h. 40 km/h. 45 km/h. 50 km/h. - AEB Inter-urban: According to the requirements of protocol 3.10.7.4.3.3, provide the brake pedal actuation speed and control, which can be used as the benchmark for equipment control and adjustment during the test. - According to the requirements of protocol 2.4.3.2.5, provide the score for all grid points per test speed for AEB Inter-Urban. (1) AEB Inter-Urban : Predicted score for CCRs-FCW Test Speed; -50 %; -75 %; 100 %; +75 %; +50 %. 30 km/h. 35 km/h. 40 km/h. 45 km/h. 50 km/h. 55 km/h. 60 km/h. 65 km/h. 70 km/h. 75 km/h. 80 km/h. (2) AEB Inter-Urban : Predicted score for CCRm-AEB Test Speed; -50 %; -75 %; 100 %; +75 %; +50 %. 30 km/h. 35 km/h. 40 km/h. 45 km/h. 50 km/h. 55 km/h. 60 km/h. 65 km/h. 70 km/h. 75 km/h. 80 km/h. (3) AEB Inter-Urban : Predicted score for CCRm-FCW Test Speed; -50 %; -75 %; 100 %; +75 %; +50 %. 50 km/h. 55 km/h. 60 km/h. 65 km/h. 70 km/h. 75 km/h. 80 km/h. - AEB Vulnerable Road User: - According to the requirements of protocol 3.11.6.4.5, provide the test speed of stop testing when the manufacturer predicts no performance of the system in each situation test as the reference information for assessment execution. - According to the requirements of protocol 3.11.6.4.6, provide the test situation to predict the collision speed.☐Provide;☐Not provide. Test Scenarios; Car-to-Pedestrian Farside Adult CPFA; Car-to-Pedestrian Nearside Adult; Car-to-Pedestrian Nearside Child CPNC. CPNA-25; CPNA-75. Predicted Collision Speed (km/h); Test Speed; Predicted collision speed (Provided by the manufacturer); Test Speed; Predicted collision speed (Provided by the manufacturer); Test Speed; Predicted collision speed (Provided by the manufacturer); Test Speed; Predicted collision speed (Provided by the manufacturer). Day; Night; Day; Night. 20; 20; 20; 20. 25; 25; 25; 25. 30; 30; 30; 30. 35; 35; 35; 35. 40; 40; 40; 40. 45; 45; 45; 45. 50; 50; 50; 50. 55; 55; 55; 55. 60; 60; 60; 60. Target speed (km/h); 8; 5; 5; 5. Visual Obstruction; None; None; None; Yes. Impact location; 50%; 25%; 75%; 50%. Test Scenarios; Car-to-Pedestrian Longitudinal Adult CPLA-50; Car-to-Bicyclist Nearside Adult CBNA-50; Car-to-Bicyclist Longitudinal Adult CBLA-50. Predicted Collision Speed (km/h); Test Speed; Predicted collision speed (Provided by the manufacturer); Test Speed; Predicted collision speed (Provided by the manufacturer); Test Speed; Predicted collision speed (Provided by the manufacturer). Day; Night. 20; 20; 25. 25; 25; 30. 30; 30; 35. 35; 35; 40. 40; 40; 45. 45; 45; 50. 50; 50; 55. 55; 55; 60. 60; 60. Target speed (km/h); 5; 15; 15. Visual Obstruction; None; Yes; None. Impact location; 50%; 50%; 50%. Test Scenarios; Car-to-Pedestrian Longitudinal Adult CPLA-25; Car-to-Bicyclist Longitudinal Adult CBLA-25. Predicted time of FCW; Test Speed; Predicted time of TTCFCW (Provided by the manufacturer); Test Speed; Predicted time of TTCFCW (Provided by the manufacturer. Day; Night. 50; 50. 55; 55. 60; 60. 65; 65. 70; 70. 75; 75. 80; 80. Target speed (km/h); 5; 20. Visual Obstruction; None; None. Impact location; 25%; 25%. (3) According to the requirements of protocol 3.11.2.3.1, the front bumper contour (or profile) of the tested vehicle shall be provided. When measuring, the width shall be reduced by 50 mm at the outermost edges on both sides. It shall be formed by connecting the evenly divided seven parallel lines with the front of the vehicle. ☐ Provided；☐ Not provided Vehicle Width; mm. Coordinate (mm); F; E; D; A; B; C. X; 0. Y; 0. NOTE: Using mathematical quadrants to represent the sign (positive or negative) of a value. Vehicle Width. Schematic diagram of the front bumper of the test vehicle

Vehicles will be considered as having limited rear space when the normal CRSs recommended by the manufacturer cannot be installed with the front seats in the test position.

“The low risk deployment frontal airbag" means the airbag can remain active when installing a rearward facing child restraint system.